# Multi-Rig 360° Stereo Deployment for Live Events

## Overview

This document analyzes the physical footprint, audience impact, and multi-rig deployment strategies for capturing concerts, Broadway shows, and other live events using 360° stereo camera rigs. It covers single-rig placement, multi-viewpoint systems for teleportation between positions, the infrastructure required for synchronized multi-rig capture, and optimized partial-coverage rig configurations that reduce costs by 20-48% for edge positions.

Key optimization: Not all deployment positions require full 360° coverage. Rigs mounted against walls, on balconies, or overhead can use 4-6 cameras instead of 8, saving significant hardware, storage, and processing costs while maintaining quality in captured directions.

See also: Partial Rig Variations Specification for detailed technical specifications of 360°, 270°, 180°, and hemisphere rig configurations.

## Single Rig Physical Footprint

### Core Rig Dimensions

Rig Unit Only: - Diameter: 300-400mm (12-16 inches) - Height: 150-200mm (6-8 inches) - Weight: 2-4kg (4-9 lbs) without cameras - Total weight with cameras: 4-6kg (9-13 lbs)

Mounted Configuration:

The rig requires mounting infrastructure that significantly increases the footprint:

Option 1: Tripod Mount (Most Common)
- Footprint: ~600mm diameter circle (tripod legs)
- Total height: 1.5-2m from ground (eye-level viewing)
- Stability: Good, requires sandbag weights
- Portability: Medium

Option 2: Monopod/Pole Mount
- Footprint: Single point, ~100mm base
- Total height: 1.5-2m from ground
- Stability: Requires guy-wires or weighted base
- Portability: High

Option 3: Overhead Rigging
- Footprint: Zero floor space
- Height: Suspended from ceiling/truss
- Stability: Excellent (no vibration)
- Portability: Low, requires venue infrastructure

Option 4: Custom Stand
- Footprint: ~400mm square base
- Total height: Adjustable 1-2m
- Stability: Excellent with proper design
- Portability: Medium

### Visual Obstruction Analysis

Horizontal Obstruction:

The rig creates a circular obstruction of approximately 400mm diameter at mounting height.

From audience member’s perspective: - At 5m distance: Rig appears ~4.6° wide (minimal obstruction) - At 10m distance: Rig appears ~2.3° wide (barely noticeable) - At 20m distance: Rig appears ~1.1° wide (negligible)

Vertical Obstruction:

Key consideration: Mounting height

Scenario 1: Eye-level mount (1.6m height)
- Obstructs view for anyone directly behind within 3-5m
- Impact: HIGH for seated audiences
- Recommendation: Avoid for ticketed seating areas

Scenario 2: Above-head mount (2.5m height)
- Minimal obstruction for standing audiences
- Slight obstruction for balcony/upper seating looking down
- Impact: LOW to MEDIUM
- Recommendation: Good for general admission concerts

Scenario 3: Overhead suspension (4m+ height)
- No obstruction for ground-level audiences
- Cameras may be too far for intimate perspective
- Impact: MINIMAL
- Recommendation: Best for Broadway/theater with rigging

### Sight Line Analysis for Typical Venues

#### Concert Venue (3,000 capacity)

Floor Plan View:

        [STAGE]
    ═════════════════

Row 1:  X X X X X X X    ← Premium seats (2m from stage)
Row 5:  X X X X X X X    ← 6m from stage
Row 10: X X X X X X X    ← 12m from stage

Rig Placement Options:

Position A: Front-center (4m from stage, 2.5m height)
  - Obstructs: Rows 1-3 center seats (6-10 seats)
  - Benefits: Best stage view
  - Verdict: HIGH IMPACT - avoid

Position B: Side-stage (2m from stage edge, 2m height)
  - Obstructs: Side-view seats (2-4 seats)
  - Benefits: Unique angle, minimal center impact
  - Verdict: MEDIUM IMPACT - acceptable

Position C: Elevated platform at sound booth (25m from stage, 3m height)
  - Obstructs: Nobody (behind audience)
  - Benefits: Zero impact, overview perspective
  - Verdict: MINIMAL IMPACT - ideal for overview

Position D: Suspended from ceiling (center, 5m height)
  - Obstructs: Nobody
  - Benefits: Perfect center position
  - Verdict: NO IMPACT - requires venue support

#### Broadway Theater (1,000 capacity)

Side View:

[STAGE]
   |
   | Orchestra Pit (2m below stage)
   ↓
[Rig Position 1] ← Stage level, 3m from front

   Row A (2m from rig)  ← Premium orchestra seats
   Row E (6m from rig)
   Row M (12m from rig)

[Mezzanine Level] (4m above orchestra)

[Rig Position 2] ← Suspended from proscenium

[Balcony Level] (8m above orchestra)

[Rig Position 3] ← Balcony rail mount


Recommended Positions:

Position 1: Orchestra center aisle (eye-level)
  - Impact: Obstructs 0 seats if in aisle
  - Requires: Aisle placement permission
  - View: Authentic audience perspective

Position 2: Proscenium suspension (4m high)
  - Impact: Zero obstruction
  - Requires: Theater rigging crew
  - View: Elevated, professional angle

Position 3: Balcony rail
  - Impact: Obstructs balcony front row sight line
  - Requires: Rail mounting bracket
  - View: High overview perspective

### Cables & Infrastructure Visibility

Beyond the rig itself, consider:

Power cables: - 8 cameras = 8+ cables to power distribution - Risk: Trip hazard if floor-mounted - Solution: Overhead cable runs, cable ramps, gaff tape

Data cables: - USB or Ethernet to central recorder - May require 5-10m cable runs - Solution: Wireless transmission (adds latency) or tidy cable management

Control station: - Computer/recorder nearby (within 5m typically) - Requires: Small table/rack space - Footprint: 600mm × 600mm area - Can be hidden behind equipment cases

## Rig Configuration Optimization

### The Partial Coverage Opportunity

Multi-rig deployments often place cameras in positions where full 360° coverage is wasteful:

| Position Type | Wasted Coverage | Solution |
| --- | --- | --- |
| Balcony rail | 180° behind = balcony wall | Use 180° (4-camera) rig |
| Against wall | 90-180° behind = wall | Use 180-270° rig |
| Ceiling mount | 180° upward = ceiling/lights | Use hemisphere (4-6 camera) rig |
| Proscenium | 180° behind = backstage | Use 180° (4-camera) rig |
| Corner position | 90° behind = two walls | Use 270° (6-camera) rig |

### Rig Type Configurations

Full Coverage (360°) - 8 cameras: - Cost: ~$5,000 (GoPro configuration) - Storage: 360 GB/hour - Use for: Floor center, photo pit, crowd positions - Coverage: Complete 360° horizontal stereo

Three-Quarter Coverage (270°) - 6 cameras: - Cost: ~$3,800 (24% savings) - Storage: 270 GB/hour - Use for: Corners, side-stage positions - Coverage: 270° stereo + 90° static panorama fallback

Half Coverage (180°) - 4 cameras: - Cost: ~$2,600 (48% savings) - Storage: 180 GB/hour - Use for: Balcony rails, walls, proscenium - Coverage: 180° stereo + 180° static panorama fallback

Hemisphere (Down-Facing) - 4-6 cameras: - Cost: ~$2,600-3,800 (24-48% savings) - Storage: 180-270 GB/hour - Use for: Ceiling suspension, overhead views - Coverage: 180° hemisphere downward only

### Uncaptured Direction Solutions

For partial rigs, uncaptured directions are handled via:

- Static Panorama (Recommended): Pre-captured 360° still image fills gaps

  - Captured before show (5 min per position)

  - Seamless blend from live stereo to static mono

  - Maintains spatial context

  - Cost: $400 one-time (Insta360 X3 for all rigs)

- Visual Boundary: Artistic frame showing “live window”

  - Clear communication of coverage limits

  - Branding opportunity

  - Cost: $0 (design only)

- Hybrid Backup: Add one 360° camera for low-res mono coverage

  - Complete 360° coverage (varying quality)

  - Cost: +$450 per rig

See: Partial Rig Variations Spec for detailed technical implementation, pipeline adaptations, and fallback solution comparisons.

### Decision Matrix: Choosing Rig Type by Position

Position Assessment:

1. Is there a wall/ceiling/obstruction behind?
   YES → Consider 180° or 270° rig
   NO → Consider 360° rig

2. Is this a premium "main viewpoint" position?
   YES → Use 360° for maximum flexibility
   NO → Partial rig acceptable

3. What % of coverage is interesting content?
   >80% → 360° rig
   60-80% → 270° rig
   <60% → 180° rig

4. Is position overhead/ceiling-mounted?
   YES → Use hemisphere rig
   NO → Use horizontal config

5. Budget constraints significant?
   YES → Maximize partial rigs
   NO → Use 360° for flexibility

## Multi-Rig Deployment Strategy

### Concept: Multi-Viewpoint VR Experience

Instead of a single fixed perspective, deploy multiple synchronized rigs throughout the venue. Users can teleport between viewpoints while watching in VR, choosing their preferred perspective.

User Experience:

User wearing VR headset sees:

  Current View: Front Row Center

  [Hotspots visible in VR:]
  → "Teleport to Stage Left"
  → "Teleport to Balcony Overview"
  → "Teleport to Behind Drummer"

  User selects hotspot → Smooth fade transition → New 360° viewpoint

  All positions maintain full 360° stereo vision

### Optimal Rig Quantity by Venue Type

#### Small Venue (500 capacity club)

Recommended: 3 rigs

[STAGE]
    ═══════

Rig 1: Front Center (2m from stage, audience POV)
Rig 2: Stage Right (1m from stage edge, performer POV)
Rig 3: Rear Center (8m from stage, room overview)

Coverage: 3 distinct perspectives
Infrastructure: Manageable for single operator
Data rate: 2.4 Gbps total (300 MB/s)
Cost impact: 3× rig cost (~$15K for GoPro setup)

#### Medium Venue (3,000 capacity arena)

Recommended: 5-7 rigs (optimized configuration)

[STAGE]
        ═════════════

Rig 1: Front Left (near stage) → 360° rig
Rig 2: Front Center (premium seat view) → 360° rig
Rig 3: Front Right (near stage) → 360° rig
Rig 4: Mid-house Left (side perspective) → 270° rig
Rig 5: Mid-house Center (sound booth area) → 360° rig
Rig 6: Mid-house Right (side perspective) → 270° rig
Rig 7: Overhead/Balcony (bird's eye) → 180° or Hemisphere rig

Coverage: Comprehensive venue coverage
Infrastructure: Requires dedicated crew (2-3 people)

Optimized cost comparison:
- All 360° rigs: 56 cameras, $35,000, 700 MB/s
- Mixed configuration: 44 cameras, $27,800, 550 MB/s
- Savings: $7,200 (21%), 150 MB/s less data

#### Large Venue (10,000+ capacity stadium)

Recommended: 8-12 rigs (optimized configuration)

Strategic positions:
- Floor front (3 rigs): Left, Center, Right close to stage → 360° rigs
- Floor mid (2 rigs): House left/right at mix position → 360° rigs
- Floor rear (1 rig): Back of venue overview → 180° rig (wall behind)
- Stage positions (3 rigs): On-stage perspectives → 270° rigs (if permitted)
- Elevated (3 rigs): Balcony levels, scaffolding → 180° or Hemisphere rigs

Coverage: Full venue, multiple height levels
Infrastructure: Dedicated crew of 4-6 people

Optimized cost comparison:
- All 360° rigs: 96 cameras, $60,000, 1,200 MB/s
- Mixed configuration: 76 cameras, $48,000, 950 MB/s
- Savings: $12,000 (20%), 250 MB/s less data

#### Broadway Theater (1,000 capacity)

Recommended: 4-5 rigs (optimized configuration)

[STAGE]
   ↓

Rig 1: Orchestra Row F Center (audience perspective) → 360° rig
Rig 2: Orchestra Row A Left (front row angle) → 360° rig
Rig 3: Mezzanine Center (elevated view) → 180° rig (looking at stage)
Rig 4: Balcony Center (upper level) → 180° rig (wall behind)
Rig 5: On-stage (performer POV, if show permits) → 270° rig (limited backstage)

Coverage: Multiple seating tiers
Infrastructure: Requires theater technical crew

Optimized cost comparison:
- All 360° rigs: 40 cameras, $25,000, 500 MB/s
- Mixed configuration: 26 cameras, $16,400, 325 MB/s
- Savings: $8,600 (34%), 175 MB/s less data

## Multi-Rig Technical Requirements

### Synchronization

Critical: All rigs must capture frames at exactly the same time.

Genlock System:

Master Clock
    ↓
    ├─→ Rig 1 (all 8 cameras synced)
    ├─→ Rig 2 (all 8 cameras synced)
    ├─→ Rig 3 (all 8 cameras synced)
    ├─→ Rig 4 (all 8 cameras synced)
    └─→ Rig 5 (all 8 cameras synced)

Total synchronized cameras: 5 rigs × 8 cameras = 40 cameras

Sync method options:
1. Wired genlock signal (professional, most reliable)
2. Network time protocol (NTP) with GPS sync
3. Wireless timecode (Tentacle Sync or similar)
4. SMPTE timecode from venue audio system

Recommended Solution: - Timecode generator (Tentacle Sync Studio or similar) - Wireless timecode receivers at each rig - ±1 frame accuracy (33ms @ 30fps) - Cost: ~$200 per rig + $500 master unit

### Network Infrastructure

For live monitoring and control of multiple rigs:

Network Topology:

[Internet] (optional, for streaming)
    ↓
[Main Router/Switch] 10 Gbps backbone
    ↓
    ├─→ [Rig 1 Recorder] 1 Gbps
    ├─→ [Rig 2 Recorder] 1 Gbps
    ├─→ [Rig 3 Recorder] 1 Gbps
    ├─→ [Rig 4 Recorder] 1 Gbps
    ├─→ [Rig 5 Recorder] 1 Gbps
    └─→ [Master Control Station] 1 Gbps

Allows:
- Remote monitoring of all rigs
- Centralized start/stop recording
- Live preview streams
- Status monitoring (storage, battery, sync)

Equipment needed: - Managed network switch (10 Gbps): $500-1,000 - CAT6 cables (up to 50m runs): $200 - Network-attached storage (NAS) for backup: $2,000-5,000

### Data Storage Scaling

Storage requirements scale linearly with rig count:

Single rig: 100 MB/s (8 cameras × 12.5 MB/s each)
5 rigs: 500 MB/s total
10 rigs: 1,000 MB/s total = 1 GB/s

For a 2-hour show:

1 rig:  720 GB
5 rigs: 3.6 TB
10 rigs: 7.2 TB

Recommended storage strategy:
- Local NVMe SSD at each rig (2TB per rig)
- Network backup to central NAS (optional during capture)
- Post-show transfer to central server for processing

Cost scaling: - 2TB NVMe SSD per rig: ~$200 × number of rigs - Central 20TB NAS: ~$3,000-5,000 - Backup drives: 2× capacity, ~$500 per 10TB

### Power Distribution

Per rig power consumption: ~80-100W

5 rigs × 100W = 500W total
10 rigs × 100W = 1,000W total

Power solutions:

Option 1: AC power distribution
- Run AC to each rig location
- Requires venue power access
- No runtime limit
- Cables may be trip hazard

Option 2: Battery packs per rig
- 150Wh battery = ~90 min runtime per rig
- Fully portable, no cables
- Limited by show duration
- Cost: ~$200 per battery × number of rigs

Option 3: Hybrid
- AC where possible (fixed positions)
- Battery for mobile/stage positions
- Best flexibility

Recommended:
- Use venue AC power for fixed positions
- Battery backup for reliability
- Hot-swap batteries for shows >2 hours

### Crew Requirements

Minimal setup (1-3 rigs): - 1 technical operator - Setup time: 1-2 hours - Can manage monitoring during show

Medium setup (4-7 rigs): - 2-3 technical operators - Setup time: 3-4 hours - Dedicated monitor operator during show

Large setup (8+ rigs): - 4-6 technical operators - Setup time: 4-6 hours - Multiple monitor operators - Dedicated troubleshooting crew member

## Placement Strategies by Venue Type

### Rock/Pop Concert Arena

Priorities: Energy, crowd atmosphere, multiple perspectives

Ideal positions:

- Photo Pit (Front center, 2m from stage, 1.5m height)

  - Captures: Performer close-ups, stage energy

  - Obstruction: Minimal (photographers already there)

  - Permission: Requires venue/artist approval

  - Rig type: 360° (premium viewpoint)

- Front of House (25m from stage, 2.5m height)

  - Captures: Professional mixer perspective

  - Obstruction: Zero (behind audience)

  - Permission: Easy (sound booth area)

  - Rig type: 180° (wall/booth behind)

- Stage Left/Right (On riser at stage edge)

  - Captures: Side stage view, performer interactions

  - Obstruction: None if on elevated platform

  - Permission: Requires production approval

  - Rig type: 270° (limited backstage interest)

- Crowd Center (15m from stage, 2m height on platform)

  - Captures: Audience immersion, crowd energy

  - Obstruction: Medium (requires small platform)

  - Permission: Requires GA floor space allocation

  - Rig type: 360° (full immersion)

- Upper Level (Balcony rail, 8m above floor)

  - Captures: Overview, venue scale

  - Obstruction: Minimal

  - Permission: Easy if venue has balcony

  - Rig type: 180° (balcony wall behind)

### Broadway Theater

Priorities: Intimacy, sightlines, artistic perspective

Ideal positions:

- Orchestra Center (Row H center aisle, 1.5m height)

  - Captures: Traditional theater audience perspective

  - Obstruction: Zero if mounted in aisle

  - Permission: Requires theater management approval

  - Note: This is the “premium seat” view

  - Rig type: 360° (premium position)

- Front Row Offset (Row A, left/right aisle, 1.3m height)

  - Captures: Intimate performer perspective

  - Obstruction: Minimal

  - Permission: Moderate difficulty

  - Rig type: 360° (full audience immersion)

- Mezzanine Front (Mezzanine Row A, 4m above orchestra)

  - Captures: Elevated view of stage and full set

  - Obstruction: Only for mezzanine front row

  - Permission: Easy

  - Rig type: 180° (looking forward at stage only)

- Proscenium Mount (Suspended from arch, 5m high, center)

  - Captures: Perfect framing of stage

  - Obstruction: Zero

  - Permission: Difficult (requires rigging crew)

  - Rig type: 180° (backstage not interesting) or Hemisphere (looking down)

Not recommended: - On-stage placement (breaks fourth wall) - Orchestra pit (unless documenting musicians)

### Classical Concert / Symphony

Priorities: Acoustics, conductor view, instrument sections

Ideal positions:

- Conductor’s Perspective (On-stage behind conductor podium, 2m height)

  - Captures: Looking at orchestra, immersive musician view

  - Obstruction: None (on stage)

  - Permission: Requires orchestra approval

  - Note: Most unique perspective

- Front Center Audience (Row 10 center, 1.5m height)

  - Captures: Traditional concert hall experience

  - Obstruction: Minimal if in aisle

  - Permission: Moderate

- Balcony Center (First balcony front row)

  - Captures: Full hall acoustic sweet spot

  - Obstruction: Minimal

  - Permission: Easy

- Stage Left/Right (Side boxes or stage level)

  - Captures: Sectional perspective (strings vs brass)

  - Obstruction: Low

  - Permission: Moderate

### Sporting Event (Arena/Stadium)

Priorities: Action following, multiple angles, replay value

Ideal positions:

- Courtside/Rinkside (2m from playing surface)

  - Captures: Player-level intensity

  - Obstruction: May block premium seats

  - Permission: Difficult (premium location)

- Behind Goal/Net (Elevated, 3m high)

  - Captures: Scoring action perspective

  - Obstruction: Low if elevated

  - Permission: Moderate

- Center Court/Ice (Upper level, suspended if possible)

  - Captures: Broadcast angle, full field view

  - Obstruction: Zero if suspended

  - Permission: Easy (press box area)

- Corner Positions (Mid-level, 4 corners)

  - Captures: Strategic overview

  - Obstruction: Low

  - Permission: Moderate

## Audience Impact Mitigation Strategies

### Strategy 1: Aisle Mounting

Place rigs in aisles rather than blocking seating: - Uses existing walkways - Zero seat loss - May require narrower rig design or offset mounting - Requires venue approval (fire code compliance)

### Strategy 2: Elevated Platforms

Small platforms (500mm × 500mm × 500mm high): - Raises rig above seated audience heads - People can see under/around it - Requires ~1 seat space worth of floor area - More stable than tripods

### Strategy 3: Overhead Suspension

Suspend from venue infrastructure: - Requires rigging points rated for load - Professional riggers needed - Zero floor obstruction - Most expensive option (~$500-1,000 per suspension point)

### Strategy 4: “Dead Space” Utilization

Use venue spaces that don’t impact ticketed audience: - Behind sound booth - In lighting towers - On stage (if production allows) - In press boxes - On balcony railings

### Strategy 5: Transparent Communication

Work with venue to: - Identify “obstructed view” seats already selling at discount - Place rigs in those locations - Further discount those seats or comp them - Invite those ticket holders to exclusive VR preview

### Strategy 6: Compact Rig Design

Optimize rig for minimal profile: - Use slim-profile cameras (machine vision vs. action cameras) - Vertical mounting (tall, narrow vs. wide) - Matte black finish (less visually obvious) - Reduces apparent size by 30-40%

## Viewpoint Switching User Experience

### Interface Design (VR Headset)

Method 1: Gaze-based Hotspots

User looks around in 360°
→ Semi-transparent markers appear at other rig locations
→ User gazes at marker for 2 seconds
→ Smooth fade transition (1 second)
→ New viewpoint loads
→ User maintains head orientation (looking same direction)

Method 2: Controller Menu

User presses menu button
→ Overhead map of venue appears
→ Icons show all available viewpoints
→ User selects with controller
→ Instant transition or fade
→ Landing orientation: facing stage/action

Method 3: Automatic Switching

AI director chooses viewpoint based on:
→ Current action (score in sports, solo in concert)
→ User viewing history
→ Dramatic timing
→ User can override at any time

### Technical Implementation

Live Streaming Multi-Rig:

Extremely bandwidth-intensive. For 5 rigs:

5 rigs × 8K stereo × 30fps × compression = ~150 Mbps per rig

Total: 750 Mbps downstream to viewer
Impractical for most consumer internet

Solutions:
1. Lower resolution per viewpoint (4K instead of 8K)
2. Adaptive bitrate (only stream active viewpoint at full quality)
3. Pre-buffer adjacent viewpoints at lower quality

Recommended for Live: Stream only 2-3 viewpoints at full quality, viewer chooses at start.

Post-Produced Multi-Rig:

Full quality available, user switches at will:

All 5 rigs processed to 8K stereo
→ Packaged in VR app with hotspot navigation
→ User downloads entire experience (~50-100GB)
→ Local playback, instant viewpoint switching
→ Best quality, zero latency

## Cost Analysis: Multi-Rig Deployments

### 3-Rig Setup (Small Venue)

| Component | Quantity | Unit Cost | Total |
| --- | --- | --- | --- |
| Camera rigs (GoPro-based) | 3 | $5,000 | $15,000 |
| Timecode sync system | 1 | $1,200 | $1,200 |
| Network switch & cables | 1 | $800 | $800 |
| Storage (2TB SSD per rig) | 3 | $200 | $600 |
| Mounting hardware | 3 | $400 | $1,200 |
| Power distribution | 1 | $500 | $500 |
| Subtotal |  |  | $19,300 |
| Crew (setup + show) | 2 people | $500/day | $1,000 |
| Total per event |  |  | $20,300 |

### 7-Rig Setup (Medium Venue)

Configuration A: All 360° Rigs (Original)

| Component | Quantity | Unit Cost | Total |
| --- | --- | --- | --- |
| Camera rigs (GoPro-based) | 7 × 360° | $5,000 | $35,000 |
| Timecode sync system | 1 | $1,500 | $1,500 |
| Network switch & cables | 1 | $1,500 | $1,500 |
| Storage (2TB SSD per rig) | 7 | $200 | $1,400 |
| Mounting hardware | 7 | $400 | $2,800 |
| Power distribution | 1 | $800 | $800 |
| Central NAS storage | 1 | $4,000 | $4,000 |
| Subtotal |  |  | $47,000 |
| Crew (setup + show) | 3 people | $600/day | $1,800 |
| Total per event |  |  | $48,800 |

Configuration B: Optimized Mixed Rigs (Recommended)

| Component | Quantity | Unit Cost | Total |
| --- | --- | --- | --- |
| 360° rigs (floor positions) | 3 | $5,000 | $15,000 |
| 270° rigs (side positions) | 2 | $3,800 | $7,600 |
| 180° rigs (balcony/wall) | 2 | $2,600 | $5,200 |
| Insta360 X3 (static panoramas) | 1 | $400 | $400 |
| Timecode sync system | 1 | $1,500 | $1,500 |
| Network switch & cables | 1 | $1,500 | $1,500 |
| Storage (reduced per rig) | 7 | $150 avg | $1,050 |
| Mounting hardware | 7 | $400 | $2,800 |
| Power distribution | 1 | $800 | $800 |
| Central NAS storage | 1 | $3,000 | $3,000 |
| Subtotal |  |  | $38,850 |
| Crew (setup + show) | 3 people | $600/day | $1,800 |
| Total per event |  |  | $40,650 |

Savings with optimized configuration: $8,150 (17% reduction) Ongoing savings: 21% less storage, 21% less processing time per event

### Amortization Over Multiple Events

If producing a series of events:

All 360° Rigs (Configuration A):

7-rig system cost: $47,000 (one-time)
Crew cost per event: $1,800
Storage/processing per event: $500

Event 1: $48,800 total ($48,800 per event)
Event 5: $56,000 total ($11,200 per event)
Event 10: $65,000 total ($6,500 per event)
Event 20: $83,000 total ($4,150 per event)

Optimized Mixed Rigs (Configuration B - Recommended):

7-rig system cost: $38,850 (one-time)
Crew cost per event: $1,800
Storage/processing per event: $395 (21% less)

Event 1: $40,650 total ($40,650 per event)
Event 5: $48,430 total ($9,686 per event)
Event 10: $57,800 total ($5,780 per event)
Event 20: $76,540 total ($3,827 per event)

Total savings over 20 events: $6,460 (8% overall reduction)

Break-even: After ~10-15 events, equipment cost is amortized. Optimized configuration maintains cost advantage throughout lifecycle.

## Distribution Strategy

### Option 1: VR App Download

Users download app with all viewpoints: - File size: 10-20GB per hour of content (all viewpoints) - Platform: Meta Quest, SteamVR, PlayStation VR - Price point: $5-20 per event - Pros: Highest quality, unlimited viewpoint switching - Cons: Large download, requires VR headset

### Option 2: Streaming Service

Users stream selected viewpoints: - Bandwidth: 50-100 Mbps per stream - Platform: Web-based or dedicated app - Price point: $15-30 per event (pay-per-view) - Pros: No large download, accessible on multiple devices - Cons: Requires high-speed internet, limited viewpoint switching

### Option 3: Hybrid

Users stream initial view, download full experience: - Start watching immediately in primary viewpoint - Background download of other viewpoints - Full quality available after buffer period - Best user experience

## Regulatory & Venue Considerations

### Permissions Required

Venue Management: - Floor plan approval for rig placement - Load capacity approval for suspended rigs - Fire code compliance (aisle obstruction) - Insurance liability

Production/Artist: - Recording rights - Distribution rights - Creative approval of viewpoints - On-stage access (if applicable)

Union/Labor: - Camera crew union agreements (if venue is union) - Rigging crew requirements - Electrical crew for power distribution

### Insurance

Typical requirements for multi-rig deployment: - General liability: $1-2 million coverage - Equipment insurance: Full replacement value - Riggers liability (for suspended equipment)

Cost: ~$1,000-3,000 per event for coverage

## Case Study Examples

### Example 1: Indie Band at 500-capacity Club

Setup: 3 rigs - Front center (audience perspective) - Stage left (on riser) - Back of room (overview)

Impact: - Obstruction: 2 tickets lost (stage riser placement) - Crew: 1 operator - Cost: $20,000 initial + $500 per show

Result: Minimal audience impact, unique fan experience

### Example 2: Broadway Musical (8 shows/week)

Setup: 4 rigs - Orchestra Row G center aisle - Orchestra Row B left aisle - Mezzanine front center - Proscenium suspension

Impact: - Obstruction: 0 tickets lost (all in aisles or suspended) - Crew: 2 operators (rotate shifts) - Cost: $25,000 initial + $1,000 per week

Result: Zero audience impact, premium VR product

### Example 3: Stadium Concert (50,000 capacity)

Setup: 10 rigs - 3 in photo pit - 2 at FOH mix position - 3 on stage truss - 2 in upper decks

Impact: - Obstruction: ~20 tickets lost (platform positions) - Revenue loss: ~$2,000 in tickets - Crew: 5 operators - Cost: $60,000 initial + $3,000 per show

Result: Minimal impact relative to capacity, global streaming opportunity

## Recommendations Summary

### For Concert Venues:

✅ Use 5-7 rigs for medium venues (3,000-5,000 cap) ✅ Prioritize: Photo pit, FOH, balcony, one crowd position ✅ Mounting: Mix of tripods and overhead suspension ✅ Impact mitigation: Dead space placement, elevated platforms ✅ Rig optimization: - 360° rigs: Photo pit, crowd center (3 rigs) - 270° rigs: Stage sides (2 rigs) - 180° rigs: FOH booth, balcony (2 rigs) - Savings: ~$7,200 (21%)

### For Broadway/Theater:

✅ Use 3-4 rigs for traditional proscenium theaters ✅ Prioritize: Orchestra center, mezzanine, suspended proscenium ✅ Mounting: Aisle placement and rigging ✅ Impact mitigation: Zero seat loss strategy ✅ Rig optimization: - 360° rigs: Orchestra center (1-2 rigs) - 180° rigs: Mezzanine, balcony, proscenium (2-3 rigs) - Savings: ~$8,600 (34%)

### For Large Festivals/Stadiums:

✅ Use 8-12 rigs for full coverage ✅ Prioritize: Multiple height levels, on-stage if possible ✅ Mounting: Professional rigging crew required ✅ Impact mitigation: Minimal % of capacity affected ✅ Rig optimization: - 360° rigs: Floor positions, premium locations (5-6 rigs) - 270° rigs: Stage/corner positions (2-3 rigs) - 180°/Hemisphere rigs: Balconies, overhead (3-4 rigs) - Savings: ~$12,000 (20%)

### Universal Optimization Guidelines:

✅ Default strategy: Use static panorama fallback for partial rigs ✅ Pre-show checklist: Capture 360° panoramas from each partial rig position (5 min each) ✅ Processing: Blend live stereo footage with static backgrounds during post-production ✅ User experience: Subtle visual indicators when viewing uncaptured zones ✅ Long-term: Optimized configurations save 20-48% per rig while maintaining quality

## Next Steps

- Venue Scout: Walk venue with floor plan, identify placement locations

- Rig Type Selection: For each position, determine optimal rig configuration (360°, 270°, 180°, or hemisphere)

- Coverage Assessment: Identify which positions have obstructions/wasted coverage

- Sight Line Test: Use cardboard mockup to test obstruction before event

- Venue Approval: Present plan to management with impact analysis

- Crew Planning: Hire appropriate crew size for rig count

- Power Survey: Map power access at each rig location

- Network Design: Plan cable runs or wireless infrastructure

- Static Panorama Planning: Schedule pre-show time to capture fallback panoramas (if using partial rigs)

- Rehearsal: Test setup day before (if possible)

- Contingency: Backup plan if rig position becomes problematic

With careful planning and optimized rig configurations, multi-rig 360° stereo capture can provide extraordinary immersive experiences while maintaining minimal impact on live audiences and reducing deployment costs by 20-35%.

## Additional Resources

For detailed technical specifications on partial rig configurations, see: - Partial Rig Variations Specification - Complete technical specs for 360°, 270°, 180°, and hemisphere rigs, including hardware requirements, pipeline adaptations, fallback solutions, and cost-benefit analysis.

Related Documents: - 360° Stereo Camera Rig Design - Core 8-camera rig design and fundamentals - GoPro Prototype Rig BOM - Bill of materials for GoPro-based implementation - Software Pipeline Complete Workflow - Processing pipeline for stitching and VR output